Perfect — with TTI geolocation enabled and TTI MQTT → ThingsBoard (MQTT subscribe) confirmed, below is a clean, end‑to‑end, step‑by‑step implementation and test design that you can actually deploy for a 5‑sticker pilot.

I’ve written this in a technical / project‑ready style so you can reuse it as design documentation or an internal SOP.

# End‑to‑End System Design

LoRaWAN Logistics Tracking with RAK2270 + TTI + ThingsBoard CE

## 1. Final Architecture (Validated Choice)

1 RAK2270 Stickers 2 ¦ 3 v 4 RAK7268CV2 Gateways 5 ¦ 6 v 7 The Things Stack (TTI) 8 - Payload decoding 9 - TDoA / RSSI-based geolocation 10 ¦ 11 v 12 TTI MQTT Broker 13 ¦ 14 v 15 ThingsBoard CE 16 - MQTT Subscribe Integration 17 - Rule Chains (mapping & enrichment) 18 - Dashboards (Map + Telemetry) 19 - Alarms

✅ Key benefits of this design

- No custom LoRaWAN server required

- Native TTI geolocation output reused

- Scales easily beyond the 5‑sticker pilot

- Clean separation of concerns (network ↔ IoT platform)

I’ve produced a one-page, publish-ready architecture diagram that reflects your exact design decisions:

- ✅ RAK2270 LoRaWAN stickers on containers

- ✅ RAK7268CV2 gateways at multiple locations

- ✅ The Things Stack (TTI) with geolocation enabled

- ✅ TTI MQTT → ThingsBoard Community Edition integration

- ✅ Location on map, temperature, battery, alarms

## 2. Preconditions (Before Configuration)

### Gateways

- Minimum 3 gateways per location for reliable TDoA

- Gateways:

  - RAK7268CV2 registered in TTI

  - Online and time‑synchronized

  - Geographically separated (not co‑located)

### Stickers

- 5 × RAK2270

- OTAA enabled

- Configured uplink interval (e.g. every 5–15 min for test)

### Platforms

- ✅ TTI with Geolocation enabled

- ✅ ThingsBoard CE reachable from Internet

- ✅ MQTT ports open (1883 or 8883)

## 3. TTI Configuration (Network Layer)

### Step 3.1 – Create Application

- TTI Console → Applications

- ID: warehouse-logistics-app

### Step 3.2 – Register Gateways

- Register each RAK7268CV2

- Validate:

  - “Connected” status

  - Uplinks visible in Live Data

✅ Acceptance: Gateway metadata shows RSSI/SNR in uplinks

### Step 3.3 – Register 5 RAK2270 Stickers

Create devices:

1 sticker-001 2 sticker-002 3 sticker-003 4 sticker-004 5 sticker-005

Configuration:

- OTAA

- Class A

- Enabled uplinks

✅ Acceptance: All devices successfully join and send uplinks

### Step 3.4 – Payload Decoder (Mandatory)

Add JavaScript uplink decoder in TTI:

Output format requirement

1 { 2 "temperature": 22.1, 3 "humidity": 48, 4 "battery": 3.05, 5 "motion": 0 6 }

✅ This ensures ThingsBoard ingestion without extra transformation logic

### Step 3.5 – Enable Geolocation

In TTI:

- Enable location solving

- Use:

  - TDoA (primary)

  - RSSI fallback (secondary)

Expected JSON field by TTI:

1 "locations": { 2 "user": { 3 "latitude": 49.5481, 4 "longitude": 8.6693, 5 "source": "TDOA" 6 } 7 }

✅ Acceptance: Uplink metadata contains locations

## 4. ThingsBoard CE Configuration

## 4.1 ThingsBoard Device Creation

Create 5 devices:

1 sticker-001 … sticker-005

Each device:

- Type: LoRaTracker

- Copy Access Token (used logically, not directly exposed to TTI)

## 4.2 MQTT Integration (TTI → ThingsBoard)

### Step 4.2.1 – Create MQTT Integration

- ThingsBoard → Integrations → MQTT

- Mode: Subscribe

- Broker: mqtt.<your-tti-region>.cloud.thethings.network

- Port: 8883

- TLS: Enabled

- Auth:

  - Username: TTI Application ID

  - Password: TTI API Key (with Read Application Traffic)

Topic filter

1 v3/+/devices/+/up

✅ Receives uplinks from all devices

## 4.3 Rule Chain: TTI → ThingsBoard Mapping

### Step 4.3.1 – Device Routing

Rule logic:

- Extract device ID:

1 end_device_ids.device_id

- Match to ThingsBoard device name

- Forward telemetry to matching device

### Step 4.3.2 – Telemetry Extraction

From:

1 uplink_message.decoded_payload

Map directly to TB telemetry:

- temperature

- humidity

- battery

- motion

### Step 4.3.3 – Location Mapping

From:

1 uplink_message.locations.user

Map Fields:

1 latitude → latitude 2 longitude → longitude

✅ This enables ThingsBoard Map widgets

## 5. Dashboard Design

### 5.1 Logistics Map (Central View)

- Widget: Map

- Marker per sticker

- Tooltip:

  - Sticker ID

  - Temperature

  - Battery

  - Last update time

  - Location source (TDoA / RSSI)

### 5.2 Device Detail Panel

- Temperature gauge

- Battery voltage trend

- Uplink count

- RSSI/SNR history (optional)

### 5.3 Alarms

Create alarms:

| Alarm | Condition |
| --- | --- |
| Temperature High | > X °C |
| Temperature Low | < Y °C |
| Low Battery | < 2.9 V |
| Device Offline | No uplink for N hours |

## 6. Structured Test Plan (5 Stickers)

### Phase 1 – Connectivity Test

Objective: Verify network & ingestion

Steps:

- Activate all stickers

- Confirm uplinks in TTI

- Confirm device telemetry in ThingsBoard

✅ Pass: 5/5 devices visible and updating

### Phase 2 – Payload Validation

Objective: Validate sensor accuracy

Steps:

- Compare reported temperature with reference

- Slightly warm/cool 1 sticker

- Confirm telemetry change

✅ Pass: Directionally correct values

### Phase 3 – Geolocation Validation

Objective: Validate triangulation

Steps:

- Place stickers near Gateway Cluster A

- Move 2 stickers to Cluster B

- Observe map updates

- Compare expected vs reported location

✅ Pass criteria:

- Correct site/zone ≥ 80%

- Location source = TDOA for majority of uplinks

### Phase 4 – Alarm Validation

Objective: Validate operational alerts

Steps:

- Simulate temperature breach

- Disable one sticker temporarily

✅ Pass: Alarms triggered & cleared correctly

## 7. Pilot Acceptance Criteria

The pilot is successful if:

✅ ≥ 95% uplinks delivered
✅ Telemetry decoded correctly
✅ TTI geolocation visible in ThingsBoard
✅ Map dashboard usable for operations
✅ Alerts reliable

## 8. Known Limitations (Important to Document)

- Indoor triangulation accuracy may be limited

- Metal racks and walls affect RF

- LoRaWAN geolocation is best for site‑level, not meter‑level indoor tracking

## 9. Next Improvements After Pilot

- Add more gateways per site

- Reduce uplink interval dynamically

- Combine with BLE/UWB for indoor precision

- Add asset lifecycle events (arrival/departure)

## 10. I Can Provide Next (Immediately)

If you want, I can now:

- ✅ Write the exact TTI → ThingsBoard rule chain JSON

- ✅ Provide a sample MQTT uplink payload → mapped telemetry

- ✅ Convert this into a formal ISO / SOP document

- ✅ Produce a one‑page architecture diagram

Just tell me which one you want next.