Below is a step-by-step, implementation-focused design to build and test (with 5 × RAK2270 sticker trackers) a warehouse logistics tracking system using:

- LoRaWAN Gateways: RAK7268CV2 (installed at multiple sites)

- LoRaWAN Network: The Things Industries / The Things Stack (TTI/TTS)

- Trackers: RAK2270 “generic sticker” (pasted on containers)

- IoT Platform: ThingsBoard Community Edition (CE)

- Features: location (via LoRaWAN geolocation/triangulation), temperature, battery, and other telemetry

- Visualization: ThingsBoard map + dashboards + alerts

I’ll structure this like a real project plan: Architecture → prerequisites → deployment → integration → dashboards → test plan → acceptance criteria → troubleshooting.

## 1) High-Level Architecture (Data Flow)

Option A (recommended): TTS → MQTT → ThingsBoard

- RAK2270 sends uplinks → nearby RAK7268 gateways

- Gateways forward to The Things Stack

- TTS provides:

  - decoded payload telemetry (temp, battery, etc.)

  - metadata (gateway IDs, RSSI, SNR, timestamps)

  - geolocation results (if configured)

- ThingsBoard consumes uplinks via MQTT integration (or HTTP webhook)

- ThingsBoard stores telemetry + renders dashboards and maps

Option B: Use ThingsBoard Gateway (LoRaWAN connector)

- More complex, typically used when you run your own LoRa Network Server locally.
Since you are already using TTI, Option A is simpler and proven.

## 2) Key Reality Check: “Triangulation” / Geolocation Expectations

LoRaWAN “triangulation” can mean different methods:

### A) TDoA (Time Difference of Arrival) – best accuracy

- Needs multiple gateways receiving the same uplink

- Needs gateways that support fine timestamping AND proper time sync (GPS, PTP, etc.)

- Accuracy can be tens to a few hundred meters (sometimes better outdoors)

### B) RSSI-based location – more common, lower accuracy

- Uses signal strength from multiple gateways

- Accuracy often hundreds of meters to kilometers

- Indoor multipath can degrade accuracy heavily

✅ Action: For your 5-sticker test, you can still proceed and validate:

- “Which site/zone” the container is in (coarse location),

- plus temperature/battery tracking reliably.

If you want true “warehouse indoor positioning,” LoRaWAN geolocation alone may be insufficient; you’d typically combine with BLE/UWB/Wi-Fi RTT. But for multi-location logistics and yard-level tracking, LoRaWAN geolocation can be fine.

## 3) Prerequisites Checklist (Before You Start)

### Network / Gateways

- Each location has at least 2–3 gateways if you expect geolocation solving (more is better).

- Gateways are registered in TTS and show “connected”.

- Gateways have stable backhaul (Ethernet preferred).

- Confirm gateway placement: high, open, away from metal structures.

### LoRaWAN / TTS

- TTS tenant + application created

- Ability to create devices and integrations

- Payload format known (or can be decoded)

### ThingsBoard CE

- Running ThingsBoard CE (Docker/VM)

- MQTT endpoint reachable from TTS (public DNS + TLS if possible)

- Admin access to create devices, rule chains, dashboards

## 4) Step-by-Step Implementation (End-to-End)

### Step 1 — Prepare ThingsBoard CE (Base Setup)

- Install ThingsBoard CE (Docker is easiest).

- Ensure you have:

  - Public URL (or VPN routing) if TTS is cloud-hosted

  - MQTT port open (1883) or MQTT over TLS (8883) depending on setup

- Create a Tenant Admin user (if not already).

Deliverable: ThingsBoard login works; you can create devices and dashboards.

### Step 2 — Configure The Things Stack (TTS/TTI)

#### 2.1 Create Application

- In TTS Console → Applications → “Create application”

- Choose region matching your gateways/tenant.

#### 2.2 Register Gateways (RAK7268CV2)

- TTS Console → Gateways → register each gateway

- Configure packet forwarder on gateway to your TTS cluster

- Confirm uplink traffic is visible (gateway “Last seen” updates)

Deliverable: Gateways are online and receiving packets.

### Step 3 — Onboard 5 × RAK2270 Stickers in TTS

- In TTS Console → your Application → End devices → “Register end device”

- Use the RAK2270 provisioning method you have:

  - OTAA (preferred): DevEUI, AppEUI/JoinEUI, AppKey

  - ABP (not preferred): DevAddr, NwkSKey, AppSKey

- Assign consistent IDs:

  - sticker-001 … sticker-005

- Verify join/uplinks:

  - Use the TTS “Live data” view

  - Ensure each sticker sends at least one uplink

Deliverable: All 5 devices join and uplink to TTS.

### Step 4 — Decode RAK2270 Payload in TTS

To use telemetry in ThingsBoard, decode payload into JSON fields.

- In TTS Console → Application → Payload formatters

- Add Uplink decoder (JavaScript) to produce JSON such as:

1 { 2 "temperature": 21.6, 3 "battery": 3.01, 4 "humidity": 45, 5 "motion": 0 6 }

✅ If you already have RAK2270 payload spec, implement decoder accordingly.
If not, you can:

- capture raw payload (frm_payload)

- use vendor docs to map bytes

- iterate during testing

Deliverable: “Decoded payload” appears in Live Data with proper fields.

### Step 5 — Enable Location / Geolocation in TTS (If Available)

Goal: produce either:

- location.latitude, location.longitude

- or a computed location object in uplink metadata

- Ensure each uplink is received by multiple gateways

- In TTS, check whether your deployment supports geolocation solving

- Enable geolocation / location solving at application level (if supported)

- Confirm uplinks include:

  - gateway metadata array (RSSI/SNR per gateway)

  - optionally geolocation output

Deliverable: Uplinks contain either:

- geolocation result (best case), OR

- enough gateway metadata to estimate location (fallback).

### Step 6 — Create TTS → ThingsBoard Integration (MQTT Recommended)

#### 6.1 In ThingsBoard: Create Devices

- ThingsBoard → Entities → Devices

- Create 5 devices:

  - sticker-001 … sticker-005

- Copy each device’s Access Token.

#### 6.2 Choose Integration Pattern

Pattern A (simple): One gateway integration → Rule chain routes to the right device

- Use one MQTT client in ThingsBoard that receives all uplinks.

- Then map uplink end_device_ids.device_id to TB device name.

Pattern B (direct): Publish to each TB device token topic

- Requires custom routing logic on the TTS side (harder).

✅ Most practical: Pattern A (single integration + mapping inside TB).

#### 6.3 Configure MQTT ingestion

You have two typical ways:

Way 1: TTS MQTT → ThingsBoard MQTT Integration

- ThingsBoard supports MQTT integrations where TB subscribes to external broker topics.

- TTS provides MQTT with auth; TB can subscribe and process JSON.

Way 2: TTS Webhook → ThingsBoard HTTP endpoint

- More straightforward sometimes (no MQTT credential complexity).

- But you asked IoT platform design—MQTT is consistent with IoT patterns.

Deliverable: ThingsBoard receives uplink messages (raw JSON).

### Step 7 — ThingsBoard Data Mapping (Telemetry + Location)

In ThingsBoard, use Rule Chains to transform incoming TTS uplinks into:

- Telemetry:

  - temperature, battery, humidity, etc.

- Attributes:

  - devEUI, applicationId, etc.

- Location:

  - latitude, longitude for map widget

  - (or lat, lon keys; depends on your map widget config)

Recommended mapping logic:

- Extract from decoded payload: uplink_message.decoded_payload.*

- Extract from location (if provided):

  - uplink_message.locations.* or similar

- If TTS does not provide final lat/lon:

  - store gateway metadata list and show “best gateway” as location zone (coarse)

Deliverable: ThingsBoard device telemetry updates in real time.

### Step 8 — Build Dashboards (Map + Telemetry + Alerts)

Create one dashboard with:

#### 8.1 Map View (Key Requirement)

- Widget: Map (OpenStreetMap / Google Maps depending on TB config)

- Show each sticker as a marker

- Marker tooltip: temperature, battery, last seen, gateway/site

#### 8.2 Device Detail Cards

- Latest temperature gauge

- Battery voltage and/or %

- Last uplink time

- RSSI/SNR trend (optional)

#### 8.3 Alerts / Alarms

Create alarms:

- Temperature out of range (e.g., > 30°C or < 5°C)

- No uplink for X hours (device lost)

- Low battery threshold

Deliverable: Operations dashboard usable by warehouse/logistics users.

## 5) Testing Plan (5 Stickers) — Practical, Repeatable

### Test Setup

- 5 stickers on 5 containers (or test objects)

- At least 2 locations (or zones) with multiple gateways each

- A test log sheet capturing:

  - Sticker ID

  - real position (ground truth)

  - time placed/moved

  - expected site/zone

  - actual TB displayed site/zone and telemetry

### Phase 1 — Connectivity & Join (Day 1)

Goal: All devices join and send uplinks reliably.

Steps

- Power/activate each sticker

- Confirm JoinAccept in TTS live data

- Confirm uplink every configured interval

- Confirm TB receives telemetry and updates device “last seen”

Pass criteria

- 5/5 stickers appear in TB

- Telemetry fields populate correctly

- No repeated join failures

### Phase 2 — Payload Accuracy (Day 1–2)

Goal: temperature/battery values make sense.

Steps

- Put sticker at room temperature → confirm expected range

- Warm/cool one sticker slightly (safe methods) → verify change detected

- Compare battery reported vs vendor expectation

Pass criteria

- Temperature changes reflect reality (directionally correct)

- Battery values are plausible and stable

### Phase 3 — Location/Geolocation Validation (Day 2–3)

Goal: Validate “triangulation” output quality.

Steps

- Place 2 stickers near Gateway A area, 2 near Gateway B, 1 in between

- Ensure each uplink is heard by multiple gateways

- Check:

  - Does TTS produce a lat/lon?

  - If not, can you derive “best gateway” zone?

- Move stickers to another location and repeat

Pass criteria (realistic for LoRaWAN geolocation)

- Correct “site/zone” classification ≥ 80%

- If lat/lon exists: location error within an agreed radius (e.g., 200–1000m depending on environment)

### Phase 4 — Alarm Testing (Day 3)

Goal: Alerts work end-to-end.

Steps

- Simulate no uplink (shield one sticker or remove battery briefly)

- Trigger temperature threshold (if safe)

- Confirm TB alarm appears + clears properly

Pass criteria

- Alarms trigger within defined window

- Alarm notifications work (email/Teams if configured)

## 6) Acceptance Criteria (What “Success” Means)

For the 5-sticker pilot, define success as:

- Reliability: ≥ 95% uplinks received over 48 hours

- Telemetry: temperature/battery decoding correct and stable

- Location: at least zone-level accuracy (site determination)

- Usability: dashboard shows map + last known status per sticker

- Alerting: missing device + temperature alarms function correctly

## 7) Common Pitfalls & Fixes (Quick Troubleshooting)

### Problem: No geolocation / triangulation output

- Need more gateways hearing each message

- Gateways not time-synced / no fine timestamping support

- Indoor reflections / metal racks distort RF

Fix: add gateways, improve placement, validate that multiple gateways capture uplinks.

### Problem: ThingsBoard receives data but doesn’t assign to correct device

- Routing not mapping device_id → TB device

Fix: implement rule chain mapping to device name or create a lookup table.

### Problem: Temperature/battery values nonsense

- Wrong payload decoder byte order / scaling

Fix: capture raw payload + confirm against vendor docs.

## 8) What I Need From You (to make this implementation exact)

To tailor the decoder + integration topics precisely, please send (copy/paste):

- A sample uplink JSON from TTS (from “Live data”) for RAK2270

- Whether your TTS tenant supports geolocation solving (or which TTS plan/features are enabled)

- Your preferred integration method:

  - TTS MQTT → TB (MQTT subscribe), or

  - TTS Webhook → TB (HTTP)

With those, I can provide:

- the exact TTS payload decoder for RAK2270,

- the exact MQTT topics / webhook format,

- and the ThingsBoard rule chain logic (step-by-step) to map each uplink to sticker-001…005 automatically.

If you want, I can also format this as an internal SOP / ISO-style procedure (like you requested in earlier work)—short, numbered, with responsibilities and evidence logs.